AEOLUS INDUCTION TASKS ROUND 2

## TASK 1: AUTONOMOUS ARCHITECTURES

## 1. Evaluation of the 5 Build Options

So out of the 5 build options we are given, I think that the 5th build option will be the best:

Depth Camera: Intel Realsense Depth camera D435i

Flight Computer: NVIDIA Jetson Orin Nano

We are choosing this because the Jetson Orin Nano computer has high AI computing ability including good GPU power which the Raspberry Pi computer lacks. This AI computing helps in locating targets more easily, which is what we want to solve our problem (locating Odysseus).

Whereas the Intel Realsense D435i has ASIC processors which process depth internally saving a lot of GPU power. It also includes on board IMU (Inertial Measurement Unit).

## 2. Rating the Parameters and weightage.

Use Case Suitability (Weight: 30%) : To detect search targets (Odysseus) while navigating around obstacles.

Compute Capability (Weight: 25%) : Needs to run specialised AI algorithms in real time for target location and other works.

Power Consumption (Weight: 20%) : Lower power consumption preserves battery life for longer flight endurance.

Weight (Weight: 15%) : Must stay comfortably below the 2.5 kg maximum takeoff limit.

Cost (Weight: 10%) : Budget efficiency for technical club usage.

| Build Option | Compute (25%) | Power (20%) | Use Case (30%) | Weight (15%) | Cost (10%) | Weighted Score |
| --- | --- | --- | --- | --- | --- | --- |
| Build 1 (Waveshare + Jetson) | 9 / 10 | 6 / 10 | 7 / 10 | 8 / 10 | 6 / 10 | 7.55 |
| Build 2 (OAK-D + Pi 5) | 6 / 10 | 8 / 10 | 7 / 10 | 9 / 10 | 8 / 10 | 7.30 |
| Build 3 (LiDAR + Pi 5) | 5 / 10 | 4 / 10 | 6 / 10 | 5 / 10 | 4 / 10 | 5.05 |
| Build 4 (Waveshare + Pi 5) | 4 / 10 | 7 / 10 | 5 / 10 | 9 / 10 | 9 / 10 | 6.00 |
| Build 5 (RealSense + Jetson) | 10 / 10 | 7 / 10 | 10 / 10 | 8 / 10 | 5 / 10 | 8.45 |

So looking at this information as well, the Build 5 stands out to be the best build.

## TASK 2: COMMUNICATION ARCHITECTURES